WEAR RESISTANCE PERFORMANCE OF CONVENTIONAL AND NON-CONVENTIONAL WIND TURBINE BLADES WITH TiN NANO-COATING

Muhammad Hasibul Hasan1, Shugata Ahmed2, and Mohammad Yeakub Ali3

1Department of Mechanical Engineering, University of Toronto, Canada

E-mail: hasibulhasan@utoronto.ca

2Department of Mechanical Engineering, International Islamic University Malaysia,

Jalan Gombak, 53100 Kuala Lumpur, Malaysia

Email: shugataahmed@gmail.com

3Department of Manufacturing and Materials Engineering, International Islamic University Malaysia,

Jalan Gombak, 53100 Kuala Lumpur, Malaysia

Email: mmyali@iium.edu.my

Abstract

Efficiency and durability are critical issues that affect widely-adopted airfoil-power generator as a sustainable source of electrical power. Even though high wind power density can be achieved; installing wind turbines in desert condition has difficulties including thermal variation, high turbulence and sand storms. Sand blasting on turbine blade surface at high velocities causes erosion resulting turbine efficiency drop. Damage-induced erosion phenomena and aeroelastic performance of the blades needed to be investigated. Suitable coating may prevent erosion to a great extent. A numerical investigation of erosion on NACA 4412 wind turbine blade has been performed using commercial computational fluid dynamics software ANSYS FLUENT 14.5 release. Discrete phase model (DPM) has been used for modelling multi-phase flow of air and sand particles over the turbine blade. Governing equations have been solved by finite volume method (FVM). Conventional 30-70% glass fiber resin and non-conventional jute fiber composite have been used as turbine blade material. Sand particles of 0.3 mm diameter have been injected from 20° , 30°, 45°, 60° and 90° angles at 50℃ temperature. Erosion rate, wall shear stress and strain rate have been calculated for different wind velocities and impingement angles. Simulation results for higher velocities deviate from the results observed at lower wind velocities. In simulation, erosion rate is highest for 30° impingement angle at low wind velocities, which has been validated by experiment with a mean absolute error (MAE) of 5.56%. Erosion rate and wall shear stress are higher on jute composite fiber than glass fiber resin. Developed shear stress on wind turbine blade surface is highest for 20° impingement angle at all velocities. On the other hand, exerted pressure on turbine blade surface is found highest for 90° angle of attack. Experimental results, with or without Titanium nitride(TiN) nano-coating, also revealed that surface roughness augments with increasing impingement angles. Nano-coating(TiN) by RF sputtering technique reduced the surface roughness significantly as oppose to uncoated samples. Highest roughness has been observed on uncoated blade surface collided with 0.3-0.69 mm diameter brown aluminium oxide particles.

| Nomenclature |  |  |  |
| --- | --- | --- | --- |
| a | Particle impact angle ° | θ | Solid domain temperature ℃ |
| Aface | Surface area of impacted wall boundary cell mm2 | μ | Dynamic viscosity N.sm-2 |
| dp | Particle diameter mm | ρ | Density of air kgm-3 |
| g | Gravitational acceleration ms-2 | ρp | Particle density kgm-3 |
| k | Turbulence kinetic energy m2s-2 |  |  |
| mp | Particle mass flow rate kgs-1 |  |  |
| n | Material independent index |  |  |
| P | Pressure Pa |  |  |
| Re | Reynolds number |  |  |
| Rerosion | Erosion rate kgm-2s-1 |  |  |
| T | Temperature ℃ |  |  |
| u | Velocity of air ms-1 |  |  |
| up | Particle velocity ms-1 |  |  |
| vi | Particle impact velocity ms-1 |  |  |
| Greek symbols α | Volume fraction of air |  |  |
| ϵ | Turbulence kinetic energy dissipation rate m2s-3 |  |  |

1. INTRODUCTION

Combustion of bio-fuels generates greenhouse gases. Releasing greenhouse gases is one of the major reasons of atmospheric temperature rise. Moreover, amount of underground petroleum storage is limited. Hence, renewable energy, such as wind, solar, geothermal etc., is becoming popular day by day as alternative sources. Wind energy is clean, available and easy to convert to electrical energy. Wind turbines are used to generate electrical power from wind energy. However, selecting turbine blade material is a critical issue. Moreover, in desert condition air flows with sand particles at relatively high velocities. Sand blasting on turbine blade surface results significant erosion. Hence, a sharp decrement of turbine efficiency is observed with time.

# Numerous trials for selecting proper wind turbine blade materials have been remarked in the literature. Mishnaevsky et al. [1] studied the usability of timber as a turbine blade material. Their investigation represents a framework for fabrication, testing and installation of timber blades. Composite materials are also getting attentions for fabricating wind turbine blades due to their low cost and attractive properties. Polymer composites had been considered by researchers to enhance surface resistance to wear such as carbon fibers, short aramids or glass [2-7], polyetheretherketone (PEEK) [8], polyphenylene sulfide (PPS) [9], polyoxymethylene (POM) [10], polyamide (PA) [11] etc.

# Patnaik et al. [12] numerically and experimentally investigated erosion rate on zinc-aluminum alloy metal ZA-27 filled with titania TiO2 for different particle impact velocity, size, temperature and impingement angle. They observed that highest erosion rate occurs at 60° impingement angle. Erosion rate on unfilled base metal was found higher than reinforced alloy. Erosion on composite material was also investigated experimentally by Drensky et al. [13]. It was found that mass loss of the material is highest for 45° impact angle. Wind velocity, particle size and fiber orientation also have great influence on mass loss. Simulation was carried out by Fiore et al. [14] to investigate erosion on wind turbine blade for insects and sand particles impact. Maximum erosion rate was detected on blade’s low pressure side. Wind tunnel testing was conducted by Gaudern et al. [15] to figure out erosion patterns on wind turbine blade surface under varying conditions. Hamed et al. [16] also explored surface damage of turbine blade both numerically and experimentally. Their results show that erosion and surface roughness enhance with increasing impact angle and particle size. Foley et al. [17] studied erosion behavior of steel AISI 4340 and found that for the same material hardness, the erosion at impact angle of 30° was much greater than at 90°. On the other hand, Hutchings [18] reported erosion peaks at 90° for AISI 52100 steel. E. Rodríguez et al. [19] found three different wear regions based on impact angles and hardness. Higher amount of wear found for 10° and 20° impact angles at lower hardness value, while, for impact angles of 60°, 75° and 90°, amount of wear is high for higher hardness value. For impact angles of 30° and 40°, wear is almost same for all hardness values.

# An effective way to prevent erosion on wind turbine blade is to provide coating. Rico et al. [20] accomplished low wear on SAE-42 steel by depositing Al2O3-13% TiO2 coating. Liang et al. [21] substantially improved damping properties of wind turbine blade using carbon nanofiber nanocomposite coating. Nano/polymer coating is promising for preventing erosion to a great extent. Zhang et al. [22] tested silica filled transparent acrylate-based coatings and remarked that nanosilica particles improve erosion resistance properties. Fig.1 illustrates coating with different percentage of nanosilica particles.

In this paper, a static NACA 4412 single wind turbine blade under desert condition has been simulated by software and laboratory setup. Numerical investigation has been carried out by ANSYS FLUENT 14.5 release. In software, discrete phase model (DPM) has been used to simulate the multiphase flow of air and sand particles. Erosion rate, development of shear stress and strain on wind turbine blade surface have been investigated. Aerodynamic performances of turbine blade have also been explored. Experiment has been carried out using air jet erosion tester. Simulation results show good agreement with experimental data.

2. MATHEMATICAL MODEL

Discrete phase model (DPM) has been used to simulate the flow of air and sand particles. In DPM, the fluid phase is considered as continuum and solid particles are treated as discrete phase. Discrete solid particles are tracked in the flow field in a Lagrangian frame of reference, where governing equations are solved for a single particle. On the other hand, transport equations for per unit volume of continuum phase is derived, which is Eulerian approach for modeling multiphase flow.

The force balance equation for a single sand particle is following:

dupdt=FDu-up+gρp-ρρp+F								(1)

As sand particles are very small, it is considered that no gravitational force is acting on them. Hence, gravitational acceleration g=0.

The term FDu-up defines the drag force/ unit mass of sand particle where -

FD=18μCDReρpdp224											(2)

Relative Reynolds number is defined as -

Re=ρdpup-uμ										(3)

The governing equations for air is derived by Eulerian method. Continuity equation is the following:

∂αρ∂t+∇αρu=0										(4)

Conservation of momentum equation:

∂αρu∂t+∇αρuu= -α∇P+∇τ								(5)

Standard k-ε turbulence model [12] is used for turbulence flow modelling. Equations related to turbulence:

∂ρk∂t+∇ρku=∇αkμeff∇k+Gk+Gb-ρε-YM+Sk				(6)

∂ρε∂t+∇ρεu=∇αεμeff∇k+C1εεkGk+C3εGb-C2ερε2k-Rε+Sε		(7)

In equation (6) and (7) -

Gk   =	Generation of turbulence kinetic energy due to the mean velocity gradients

Gb   =	Generation of turbulence kinetic energy due to buoyancy

YM   =	Contribution of the fluctuating dilatation in compressible turbulence to the overall 	dissipation rate

Sk   =	Source term for turbulence kinetic energy

Sε   =	Source term for turbulence kinetic energy dissipation rate

The erosion model is included in DPM. Erosion rate is defined by following equation:

Rerosion=p=1NparticlesmpCdpFavinAface								(8)

Here, Cdp is particle diameter function and Fa is impact angle function.

Following boundary conditions are applied in the computational domain to solve the governing equations numerically:

Inlet: velocity of air at Y-direction, uy=uin; other velocity components, ux=uz=0; temperature of air and sand particles, Tin=50℃; mass flow rate of sand particles, m=min.

Outlet: pressure, p=pout. Atmospheric pressure is defined at domain outlet.

Walls: Fluid velocity adjacent to stationary domain walls, u=0.

Outer walls are considered thermally insulated. Hence, temperature gradient normal to the wall, ∂θ∂n=0.

3. NUMERICAL SIMULATION

3.1 Meshing and Grid Independent Test

A Hybrid mesh has been generated by ANSYS Workbench. Fig. 2 shows meshing of turbine blade surface and the fluid domain with 60° impingement angle. The grid consists of hexahedron elements. Minimum number of elements should be considered in the simulation to minimize computational time. To find out optimum number of elements, grid independent test has been carried out. Erosion rate has been observed for 1.3×105 to 1.5×105 number of elements and relative error has been calculated, which are shown in Table 1. Finally, 144828 number of cells has been selected for simulation with a relative error of 1.36%.

3.2 Solution methods

Governing equations have been solved numerically by finite volume method (FVM). Second order upwind scheme has been used for discretization. Pressure-velocity coupling equation has been solved by Semi-Implicit Method for Pressure Linked Equations (SIMPLE) algorithm, developed by Patankar and Spalding [13].

4. EXPERIMENTAL STUDY

4.1 Samples and nanocoating

Glass fiber 6 X 2 cm samples were collected from the 3mm thick turbine blades and jute reinforced epoxy resin composite samples were prepared in the laboratory. Multiple acetone wash removed dusts and gel coat of the turbine blades, if any. RF sputtering technique was adopted for the coating while target material was 99.99% Titanium. Gas flow rate for Argon(Ar) was 50 sccm and for Nitrogen(N) was 5 sccm. Duration of each deposition was 60 minutes.

4.2 Sand blasting and roughness analysis

Sand blasting machine with a blasting pressure of 50 psi (20 m/sec) and 75 psi (35 m/sec) were used to erosion rate calculation. The test rig is shown in Fig. 3. Target samples were kept 20cm away from the nozzle tip and the duration of sand blasting was 10 seconds for each sample. Brown Aluminum oxide of 0.30 to 0.70 mm particles were used as sand particles.

Sand blasted samples were analysed by Mitutoyo surftest (SJ-301) surface roughness analyser. JIS B0601(2001) method was adopted and Gauss filter was used to calculate the roughness (Ra) value.

5. RESULTS AND DISCUSSION

5.1 Simulation results

5.1.1 Erosion rate

Fig. 4 shows the contour plot of erosion on wind turbine blade surface of glass fiber resin for 60 ms-1 wind velocity and 90° impingement angle. It is observed that highest erosion rate on turbine blade surface is 1.4×10-6 kgm-2s-1.

In Fig. 5, highest erosion rate on wind turbine blade surface of glass fiber resin is plotted for different velocities and impingement angles. It is found that erosion rate elevates with increasing wind velocity for all angles. At low wind velocities, erosion rate is highest for 30° impingement angle, which is followed by 90°, 60°, 45° and 20° angles respectively. However, erosion rate at high velocities for 90° and 30° angles are same. Exceptionally, at high velocities, erosion rate for 45° angle is higher than 60° impingement angle. At 65 ms-1 wind velocity, erosion rate for 45° angle is 8×10-7 kgm-2s-1, while for 60° angle it is 6×10-7 kgm-2s-1.

In Fig. 6, erosion rate for 30° impingement angle is plotted for jute fiber composite and 30-70% glass fiber resin. It shows clearly that erosion rate on jute fiber composite blade is higher than glass fiber resin.

4.1.2 Wall shear stress

In Fig. 7, development of shear stress on wind turbine blade surface is shown by contour plot. It is visible that shear stress is very high at the edge of the blade. Hence, the possible damage from wind flow may occur at the blade edge.

Similar to erosion rate, shear stress on the blade surface also elevates with increasing wind velocity for all angles, which is observed in Fig. 8. Wall shear stress for 20° impingement angle is much higher than other angles. Exceptionally, wall shear stress for 90° angle at 65 ms-1 wind velocity is higher than 30° and 60° angles. However, at other velocities, shear stresses for 30° and 60° angles are found higher than 90° angle. At 40 ms-1 wind velocity, shear stress for 60° impingement angle is about 50 Pa, which is much higher than 30° angle. However, at higher velocities, shear stresses for 30° and 60° angles are found almost same. Lowest shear stress is developed for 45° angle at all velocities.

Shear stress is also higher on jute fiber composite blade than glass fiber resin. It is observable in Fig. 9.

4.1.3 Strain rate

In Fig. 10, alike wall shear stress, strain rate is also high at the edge of the blade. Again, from Fig. 11, it is seen that strain rate rises with the increment of wind velocity. Strain rate is found highest for 60° angle of attack at all velocities, which is followed by 30°,20°, 45° and 90° angles respectively. However, at 65 ms-1 wind velocity, same strain rate is observed for 45° and 90° angles.

4.1.4 Pressure distribution

In Fig. 12, pressure distribution on 30-70% glass fiber resin blade surface are shown for various angle of attacks has been plotted. It is perceived that highest pressure developed on blade surface declines with increasing angle. However, for 90° angle of attack, highest pressure developed on the blade suddenly rises to 29.5 KPa, which is larger than pressures developed at all other angles.

4.2 Experimental results

4.2.1 Comparison with simulation results

In Fig. 13, both experimental and simulation results of erosion rate for 5⁰, 30⁰ and 60⁰ impingement angles have been plotted. Simulation results show good agreement with experimental data for 5⁰ and 60⁰ angles. However, for 30⁰ angle, erosion rate is observed higher in experiment. The mean absolute error (MAE) is defined using following equation:

MAE=1MRerosion,pred-Rerosion,expRerosion,exp×100%						(9)

Here, M represents number of data points. Calculated MAE for 30° impingement angle is 5.56%.

4.2.2 Surface roughness

Surface roughness increases almost linearly with increasing impingement angle, which can be apprehended from Fig. 14. Surface roughness on uncoated blade surface collided with 0.3-0.69 mm diameter particles is highest, which is expected. Surface roughness is found low for finer particle sizes.

5. Conclusion

Rate of Erosion, development of shear stress and strain rate on wind turbine blade surface in desert conditions have been explored. Pressure distribution on wind turbine blade for various injection angles has also been observed. Major findings from numerical and experimental investigations are following -

1. In both simulation and experiment, erosion rate from wind turbine blade surface is found highest for 30° impingement angle.

2. Shear stress development on wind turbine blade surface is highest for 20° angle and lowest for 45° angle. Although strain rate has been found lowest for 90° impingement angle, it is also very low for  45° angle.

3. Erosion rate on glass fiber resin has been found lower than jute fiber composite. Hence, it is more suitable as a wind turbine blade material from erosion perspective.

4. Pressure distribution on turbine blade surface is highest for 90⁰ angle of attack.

5. Surface roughness increases linearly with increment of impingement angle. As expected, surface roughness observed on uncoated blade surface is higher than coated surfaces.

References

[1] Mishnaevsky, L., Freere, P., Sinha, R., Acharya, P., Shrestha, R., & Manandhar, P. (2011). Small wind turbines with timber blades for developing countries: Materials choice, development, installation and experiences. Renewable Energy, 36(8), 2128-2138.

[2] Stachowiak, G.W. and Batchelor, A.W. 2001. Engineering Tribology, 2nd, Boston: Butterworth-Heinemann.

[3] Friedrich, K. 1997. Wear Performance of High Temperature Polymers and their Composites. CRC Press, : 221–246.

[4] Briscoe, B.J. 1993. The tribology of composite materials: a preface. Adv. Compos. Tribol, 8: 3–15.

[5] Hager, A.M. and Davies, M. 1993. Short-fibre reinforced, high temperature resistant polymers for a wide field of tribological applications. Adv. Compos. Tribol, 8: 107–157.

[6] Kong, C., Bang, J., & Sugiyama, Y. (2005). Structural investigation of composite wind turbine blade considering various load cases and fatigue life. Energy, 30(11), 2101-2114.

[7] Joosse, P. A., Van Delft, D. R. V., Kensche, C., Hahn, F., Jacobsen, T., & van den Berg, R. (2000, January). Economic Use of Carbon Fibres in Large Wind Turbine Blades. In Proceedings of AIAA/ASME Wind Energy Symposium. Reno, NV.

[8] Bahadur, S. and Gong, D. 1992. The role of copper compounds as fillers in the transfer and wear behavior of polyetheretherketone. Wear, 154: 151–165.

[9] Schwartz, C.J. and Bahadur, S. 2001. The role of filler deformability, filler polymer bonding, and counterface material on the tribological behavior of polyphenylene sulfide (PPS). Wear, 251: 1532–1540.

[10] Yu, L., Yang, S., Wang, H. and Xue, Q. 2000. An investigation of the friction and wear behaviors of micrometer copper particle-filled polyoxymethylene composites. J. Appl. Polymer Sci, 19: 2404–2410.

[11] Bahadur, S. and Kapoor, A. 1992. The effect of ZnF2, ZnS and PbS fillers on the tribological behavior of nylon 11. Wear, 155: 49–61.

[12] A. Patnaik, T. G. Mamatha, S. Biswas, & P. Kumar, (2012). Damage assessment of titania filled zinc–aluminum alloy metal matrix composites in erosive environment: A comparative study. Materials & Design, 36 (2012), 511-521.

[13] Drensky, G., Hamed, A., Tabakoff, W., & Abot, J. (2011). Experimental investigation of polymer matrix reinforced composite erosion characteristics. Wear, 270(3), 146-151.

[14] Fiore, G., & Selig, M. S. A Simulation of Operational Damage for Wind Turbine Blades.

[15] N. Gaudern, A practical study of the aerodynamic impact of wind turbine blade leading edge erosion. Journal of Physics: Conference Series, 524(1) (2014).

[16] A. A. Hamed, W. Tabakoff, R. B. Rivir, K. Das & P. Arora. Turbine blade surface deterioration by erosion. Journal of Turbomachinery, 127(3) (2005), 445-452.

[17] Foley, T., & Levy, A. (1983). The erosion of heat-treated steels. Wear, 91(1), 45-64.

[18] Hutchings, I. M. (1992). Tribology: friction and wear of engineering materials.

[19] Rodríguez, E., Flores, M., Pérez, A., Mercado-Solis, R. D., González, R., Rodríguez, J., & Valtierra, S. (2009). Erosive wear by silica sand on AISI H13 and 4140 steels. Wear, 267(11), 2109-2115.

[20] Rico, A., Poza, P., & Rodríguez, J. (2013). High temperature tribological behavior of nanostructured and conventional plasma sprayed alumina-titania coatings. Vacuum, 88, 149-154.

[21] Liang, F., Gou, J., Kapat, J., Gu, H., & Song, G. (2011). Multifunctional nanocomposite coating for wind turbine blades. International Journal of Smart and Nano Materials, 2(3), 120-133.

[22] Zhang, H., Tang, L., Zhang, Z., Gu, L., Xu, Y., & Eger, C. (2010). Wear-resistant and transparent acrylate-based coating with highly filled nanosilica particles. Tribology International, 43(1), 83-91.

[23] B. E. Launder and D. B. Spalding. Lectures in Mathematical Models of Turbulence. Academic Press, London, England. 1972.

[24] S. V. Patankar and D. B. Spalding, A calculation procedure for heat, mass and momentum transfer in three-dimensional parabolic flows. International Journal of Heat and Mass Transfer, 15(10) (1972), 1787-1806.

Fig. 1. TEM micrograph of nano/polymer coating with (a) 10wt% and (b) 40wt% nanosilica particles [22]

(a)

(b)							(c)

Fig. 2. (a) wind turbine blade surface meshing; Meshing of fluid domain with 60° impingement angle - (b) cross-sectional view, (c) three dimensional view

# Table 1. Grid independence test results

| No. of elements | Rerrosion | Relative error % |
| --- | --- | --- |
| 134425 | 1.31×10-7 | 13.74 |
| 137854 | 1.38×10-7 | 7.97 |
| 144828 | 1.47×10-7 | 1.36 |
| 148710 | 1.44×10-7 | 3.47 |
| 153318 | 1.49×10-7 | ------ |

Fig. 3. Test facility for sand blasting on wind turbine blade prototype

Fig. 4. Erosion on wind turbine blade surface of glass fiber-resin for 60 ms-1 wind velocity and 90° impingement angle

Fig. 5 Erosion rate vs. wind velocity curve for glass fiber resin at different angle of attacks

Fig. 6. Rate of erosion on two different composite materials for 30° angle of attack

Fig. 7. Shear stress on wind turbine blade surface of glass fiber-resin for 65 ms-1 wind velocity and sand injection from 90° angle

Fig. 8. Wall shear stress vs. wind velocity curve for glass fiber resin at different angle of attacks

Fig. 9. Development of shear stress on two different composite materials for 30° angle of attack

Fig. 10. Strain rate on wind turbine blade surface of glass fiber-resin for 65 ms-1 wind velocity and sand injection from 60° angle

Fig. 11. Strain rate vs. wind velocity curve for glass fiber resin

Fig. 12. Highest pressure vs. injection angle curve for glass fiber resin, 35 ms-1 wind velocity, 0.3 mm diameter sand particle injection at  0.5 kgs-1 mass flow rate

Fig. 13. Comparison of experimental and simulation results for erosion rate at different angle of attacks

Fig. 14. Experimental results of surface roughness vs. angle of attacks for different coated and uncoated surfaces